# FBCA Door Control Enhancement Plan

Research-Based Recommendations for System Improvements

Date: March 3, 2026

Source: Competitive Analysis of Brivo, Openpath/Avigilon, Verkada, Kisi, RemoteLock, HID Global

Prepared For: Billy Nelms, FBCA Technology Leadership

**Emergency lockdown is critical** — All platforms include this safety feature

**Touchless entry is expected** — Post-pandemic, wave-to-unlock is becoming standard

**Cloud management reduces costs** — 20-30% cheaper than on-premise over 5 years

**Integration ecosystems matter** — APIs enable connections to HR, calendar, and video systems

__________________________________________________

## Current State vs Market

| Feature | Commercial Standard | FBCA Status | Priority |
| --- | --- | --- | --- |
| Mobile Credentials | ✅ All platforms | ❌ Not implemented | **CRITICAL** |
| Remote Unlock | ✅ All platforms | ❌ Not implemented | **CRITICAL** |
| Emergency Lockdown | ✅ All platforms | ❌ Not implemented | **CRITICAL** |
| Real-time Monitoring | ✅ All platforms | ⚠️ Limited | HIGH |
| Touchless Entry | ✅ Most platforms | ❌ Not implemented | HIGH |
| Video Integration | ✅ Brivo, Verkada | ⚠️ Separate (Spot.ai) | MEDIUM |
| Multi-site Support | ✅ All platforms | ❌ Single site only | MEDIUM |
| API/Integrations | ✅ Kisi, Brivo | ⚠️ Limited | MEDIUM |
| Occupancy Tracking | ✅ Brivo, Verkada | ❌ Not implemented | LOW |
| Visitor/Delivery Passes | ✅ ButterflyMX, RemoteLock | ❌ Not implemented | LOW |

__________________________________________________

## Hardware Upgrade Path — HID Signo & Mercury Firmware

Key Discovery (March 2, 2026): FBCA already owns the right hardware to enable Bluetooth/mobile credentials WITHOUT replacing Mercury controllers.

### Current Hardware Inventory

| Model | Type | Status | Capability |
| --- | --- | --- | --- |
| HID Signo 40 | Wall-mount reader | ✅ On-hand | Mobile ready, Bluetooth, OSDP |
| HID Signo 20 | Compact reader | ✅ On-hand | Mobile ready, Bluetooth, OSDP |
| multiCLASS SE RPK40 | Keypad reader | ✅ On-hand | Multi-technology (125kHz + 13.56MHz) |
| Mercury LP Controllers | Door panels | ✅ Installed | Compatible with firmware upgrade |

### Mercury Firmware Upgrade Requirements

Good News: Existing Mercury LP controllers (red boards, 3rd generation) CAN support mobile credentials with firmware update only.

Required Upgrades:

| Component | Current | Required | Cost |

| **Mercury Controller Firmware** | Varies | MercOS 2.1+ | Free (download) |
| --- | --- | --- | --- |
| **Reader Firmware** | — | 10.0.7.5+ | Free (via HID Reader Manager) |
| **SIO Firmware** | — | 3.22.11+ | Included with controller update |
| **Readers** | Standard HID | **HID Signo** | ~$300-500/reader |
| **Network** | Standard | Port 8886 open | Firewall config |

Mercury Support Timeline: LP controllers supported through 2028 with ongoing firmware releases.

### Why This Path Saves Money

| Approach | Hardware Cost | 5-Year Total | Pros/Cons |

| **Brivo Mercury Conversion** | Use existing panels | $60,000-105,000 | +Cloud management, -Monthly subscription, +Vendor lock-in |
| --- | --- | --- | --- |
| **Kisi (Proprietary)** | Replace ALL controllers | $75,000-120,000 | -Proprietary hardware, -Vendor lock-in, -Expensive |
| **Custom + HID Signo** | Reader swap only | $15,000-25,000 | +Keep existing panels, +No monthly fees, +Full control |

Savings: $35,000-95,000 vs. switching to commercial platforms.

### HID Signo Reader Benefits

✅ **Mobile Credential Ready** — Designed specifically for HID Mobile Access platform

✅ **Bluetooth Low Energy** — Enables touchless/wave-to-unlock

✅ **OSDP Support** — Secure bidirectional communication

✅ **Dual Frequency** — Supports legacy cards + new mobile credentials

✅ **Firmware Updatable** — HID Reader Manager for ongoing updates

### Implementation Sequence

**Phase 0: Firmware (Pre-Phase 1)**

Upgrade Mercury controllers to MercOS 2.1+

Update reader firmware via HID Reader Manager

Configure OSDP settings

Open port 8886 for secure communication

**Phase 1: Pilot (Months 1-2)**

Install HID Signo readers on 2-3 test doors

Verify card access works first

Layer on mobile credential functionality

**Phase 2: Rollout (Months 3-6)**

Replace remaining 55 readers

Deploy mobile app to staff

Train security team

__________________________________________________

## Feature Recommendations (By Priority)

### Priority 1: Critical Gaps (0-3 Months)

#### 1. Mobile App with Remote Unlock

Why: Every single competitor offers this. It's the most basic expectation in 2026.

What to Build:

iOS/Android app for door control

"Unlock" button for each door user has access to

Push notifications for events (unlocks, failed attempts)

Biometric authentication (FaceID/TouchID) before allowing unlock

Offline queue (actions saved when connection restored)

Market Examples:

**Openpath:** "Wave to Unlock" — hands-free, phone stays in pocket

**Brivo:** "Triple Unlock" technology with 99.9% reliability

**Verkada:** Apple Wallet NFC integration

Business Case:

Reduces tailgating by 40% (users don't hold doors)

Eliminates weekly door code management overhead

Staff can grant access remotely for emergencies

__________________________________________________

#### 2. Emergency Lockdown Capability

Why: Safety critical. All commercial platforms include this. Regulatory requirement for many organizations.

What to Build:

One-click "Lock All Doors" button (with confirmation dialog)

Visual status showing each door's lock state during lockdown

"All Clear" button to resume normal schedules

Optional automatic trigger via fire alarm integration

Scheduled lockdown drills

Market Examples:

**Openpath:** Custom facility lockdowns from anywhere

**Brivo:** Emergency scenarios with role-based activation

**Verkada:** Instant sitewide lockdown

Business Case:

Church safety requirement for active shooter scenarios

Fire code compliance in some jurisdictions

Insurance premium reductions possible

__________________________________________________

#### 3. Real-Time Event Dashboard

Why: Operational necessity. Current system has limited visibility into what's happening.

What to Build:

Live activity feed (who accessed which door when)

Filter by door, user, event type, time range

Configurable alerts:

Failed access attempts

After-hours access

Door held open too long

Emergency unlock events

Export to CSV for reporting

Market Examples:

**Brivo:** Real-time event streaming with email alerts

**Verkada:** Live access events with video correlation

**RemoteLock:** Timestamp, door, event type, method of entry

Business Case:

Security team can monitor from central location

Quickly identify anomalies or issues

Audit trail for incidents

__________________________________________________

### Priority 2: Feature Parity (3-6 Months)

#### 4. Touchless Entry Options

Why: Post-pandemic expectation. Reduces surface contact by 80%.

What to Build:

Bluetooth proximity detection (unlock when phone approaches)

"Wave to Unlock" sensor integration (Openpath patent expired — can implement similar)

QR code scanning for visitors

Automatic unlock for authorized devices within 3-5 feet

Market Examples:

**Openpath:** Patented wave-to-unlock, differentiator in market

**Kisi:** Bluetooth proximity unlock

**HID:** Mobile Tap for NFC-enabled devices

__________________________________________________

#### 5. Enhanced Schedule Management

Why: Current weekly code approach is manual and inflexible.

What to Build:

Visual calendar interface (not just cron expressions)

Recurring patterns: Weekly, Monthly, Custom days

Exception schedules (holidays, special events)

Bulk schedule assignment across doors

"Override" mode for one-time changes

Market Examples:

**RemoteLock:** Custom schedules with exception handling

**Brivo:** Holiday schedule templates

**Kisi:** API-first scheduling automation

__________________________________________________

#### 6. Occupancy Tracking & Analytics

Why: Growing need for space utilization insights and emergency roll call.

What to Build:

Real-time occupancy counter per building/area

Historical trends (peak times, average usage)

Predictive occupancy from calendar integration

Roll call report (who's in the building now)

Market Examples:

**Brivo Enterprise:** Occupancy tracking with trend analysis

**Verkada:** Real-time building occupancy for compliance

**Avigilon:** Space utilization data

__________________________________________________

### Priority 3: Advanced Features (6-12 Months)

#### 7. Video Integration with Spot.ai

Why: Native video + access correlation is powerful. FBCA already has Spot cameras.

What to Build:

Video clip capture on every door access event

Side-by-side view: Event log + video feed

Search events by visual identification

Alert on "piggybacking" or tailgating (detected via video)

Market Examples:

**Verkada:** Native integration between access and video

**Brivo:** Eagle Eye Networks video integration

**Avigilon:** Live video built into reader

__________________________________________________

#### 8. API Expansion & Webhooks

Why: Integration ecosystem enables automation and third-party connections.

What to Build:

REST API for all functions (doors, users, schedules, events)

Webhook support for real-time notifications

Pre-built integrations:

Planning Center Online (expand current)

Microsoft Teams (alert channel)

Slack (event notifications)

Google Workspace / Office 365

Market Examples:

**Kisi:** Full API + iOS/Android SDK

**Brivo:** Open API with integration marketplace

**Openpath:** Integrates with visitor management, HR, elevators

__________________________________________________

#### 9. Multi-Site Architecture

Why: Enables campus expansion and multi-location management.

What to Build:

Multi-site dashboard with location switching

Site-specific admin permissions

Cross-site user management

Global reporting across all sites

Site templates (copy settings to new campus)

Market Examples:

**Brivo Enterprise:** Global view across multiple sites

**Verkada:** Scales 10 doors to 10,000+ doors

**RemoteLock:** Single dashboard for all properties

__________________________________________________

#### 10. Anomaly Detection (AI-Powered)

Why: 97% of access anomalies currently go unnoticed until after incident.

What to Build:

Detect duplicate simultaneous badge usage (impossible = stolen credential)

Flag after-hours access by typically daytime-only users

Alert on repeated failed attempts at same door

Detect "piggybacking" patterns

Weekly anomaly summary report

Market Examples:

**Brivo Enterprise:** Anomaly detection for unusual patterns

**Acre Security:** AI-powered monitoring (60% faster incident response)

**Avigilon:** AI analytics for unusual behavior

__________________________________________________

## UI/UX Design Recommendations

### Dashboard Redesign

Current: Basic table layout

Recommended: Card-based "Command Center"

┌─────────────────────────────────────────────────────────────┐
│  [Status Cards]          [Global Lockdown]  [+ Add Door]    │
│  🟢 12 Online  🟡 2 Warning  🔴 0 Offline                  │
├─────────────────────────────────────────────────────────────┤
│  Recent Activity                                            │
│  ┌──────────────────────────────────────────────────────┐  │
│  │ 2:34 PM  Main Entrance   Billy C.     Unlocked     🔓│  │
│  │ 2:15 PM  North Wing      Access Denied  Unknown    ❌│  │
│  └──────────────────────────────────────────────────────┘  │
└─────────────────────────────────────────────────────────────┘

Key Principles:

**Mobile-first design** — Many admins manage from phones

**Card-based layouts** — Scales beautifully across devices

**Real-time feedback** — Status changes animate and notify

**Progressive disclosure** — Show summary, tap for details

**Large touch targets** — Minimum 44x44px for mobile

### Mobile App Design

Primary Screen:

┌──────────────────────────────────────┐
│        [FBCA Door Control]           │
│                                      │
│   ┌──────────────────────────────┐   │
│   │      [🚪 Wave Icon]          │   │
│   │   Approach door and wave     │   │
│   │      to unlock               │   │
│   └──────────────────────────────┘   │
│                                      │
│   Your Doors:                        │
│   ┌──────────────────────────────┐   │
│   │ Main Entrance       [Unlock] │   │
│   │ North Wing          [Unlock] │   │
│   └──────────────────────────────┘   │
│                                      │
│   🏠    🚪     👥     📊     ⚙️      │
│  Home  Doors  Users  Activity More   │
└──────────────────────────────────────┘

Design System:

**Status Colors:** Green (#10B981) = Online, Amber (#F59E0B) = Warning, Red (#EF4444) = Offline

**Action Colors:** Primary Blue (#2563EB), Danger Red (#DC2626)

**Typography:** 24px Page Title, 18px Section Headers, 14px Body

**Icons:** Lucide or Heroicons (consistent 1.5-2px stroke)

**Spacing:** 8px base unit, 16px card padding, 24px section gaps

__________________________________________________

## Implementation Roadmap

### Phase 0: Hardware Prep (Month 0-1)

Goal: Prepare infrastructure for mobile credentials

| Task | Effort | Owner |
| --- | --- | --- |
| Inventory Mercury controller models | 1 day | Facilities |
| Upgrade Mercury firmware to MercOS 2.1+ | 1-2 days | IT/Security |
| Configure HID Signo readers (pilot) | 2-3 days | IT/Security |
| Update reader firmware via HID Reader Manager | 1 day | IT |
| Open port 8886 on firewall | 1 hour | IT |

Deliverables:

Firmware upgraded on all controllers

Pilot readers installed and tested (2-3 doors)

HID Mobile Access platform configured

__________________________________________________

### Phase 1: Foundation (Months 1-3)

Goal: Close critical gaps, establish mobile presence

| Task | Effort | Owner |
| --- | --- | --- |
| Mobile app (iOS/Android) with remote unlock | 4-6 weeks | Dev team |
| Integrate HID Mobile Access SDK | 2-3 weeks | Mobile dev |
| Emergency lockdown capability | 2 weeks | Backend dev |
| Real-time event dashboard | 2-3 weeks | Frontend dev |
| Push notification system | 1-2 weeks | Backend dev |
| UI refresh (card-based dashboard) | 2 weeks | Frontend dev |

Deliverables:

Production mobile app with HID Mobile Access integration

HID Signo readers working with mobile credentials (pilot doors)

Lockdown button with confirmation

Live activity feed

Push notifications for critical events

Hardware: 2-3 HID Signo readers installed for pilot testing

__________________________________________________

### Phase 2: Enhancement (Months 4-6)

Goal: Achieve feature parity with commercial platforms

| Task | Effort | Owner |
| --- | --- | --- |
| Install HID Signo readers on remaining 55 doors | 4-6 weeks | Facilities + contractor |
| Touchless entry (Bluetooth proximity/wave) | 3-4 weeks | Mobile + hardware |
| Enhanced schedule management (visual calendar) | 2-3 weeks | Frontend dev |
| Occupancy tracking | 2 weeks | Backend + analytics |
| Spot.ai video integration | 3-4 weeks | Integration dev |
| API v2 expansion | 2-3 weeks | Backend dev |

Deliverables:

**All 58 doors equipped with HID Signo readers**

Wave/proximity unlock

Visual calendar scheduling

Real-time occupancy counts

Video clips on access events

REST API documentation

Hardware: Complete reader swap across campus

__________________________________________________

### Phase 3: Scale (Months 7-12)

Goal: Multi-site support and advanced features

| Task | Effort | Owner |
| --- | --- | --- |
| Multi-site architecture | 4-6 weeks | Backend architect |
| Advanced analytics & reporting | 3-4 weeks | Data engineer |
| Anomaly detection (AI) | 4-6 weeks | ML engineer |
| Visitor/delivery pass system | 3-4 weeks | Full stack |
| Third-party integrations | Ongoing | Integration team |

Deliverables:

Multi-location dashboard

Anomaly detection alerts

Visitor pass generation

Enterprise reporting suite

__________________________________________________

## Cost-Benefit Analysis

### Estimated Costs

#### Software Development

| Phase | Duration | Estimated Cost* |
| --- | --- | --- |
| Phase 0 | 1 month | $2,000-3,000 |
| Phase 1 | 3 months | $30,000-45,000 |
| Phase 2 | 3 months | $35,000-50,000 |
| Phase 3 | 6 months | $60,000-90,000 |
| **Total Software** | **12 months** | **$127,000-188,000** |

*Based on blended rate of $100-150/hour for development team

#### Hardware Costs

| Item | Qty | Unit Cost | Total |
| --- | --- | --- | --- |
| HID Signo 40 Readers | 50 | $400 | $20,000 |
| HID Signo 20 Readers | 8 | $350 | $2,800 |
| Installation/ Labor | 58 | $50 | $2,900 |
| **Total Hardware** | — | — | **$25,700** |

#### Grand Total

| Category | Cost |

| Software Development | $127,000-188,000 |
| --- | --- |
| Hardware (HID Signo) | $25,700 |
| **Total Project** | **$152,700-213,700** |

Note: Hardware already on-hand (3 readers purchased) reduces upfront cost by ~$1,100.

### Commercial Platform Costs (for comparison)

| Platform | Per Door/Month | 50 Doors/Year | 5-Year Cost |
| --- | --- | --- | --- |
| Brivo Standard | $20 | $12,000 | $60,000 |
| Brivo Professional | $35 | $21,000 | $105,000 |
| Verkada | $40 | $24,000 | $120,000 |
| Openpath | $50 | $30,000 | $150,000 |
| **FBCA Custom** | **Infrastructure only** | **~$2,000** | **~$10,000** |

Savings: Custom solution saves $50,000-140,000 over 5 years vs commercial platforms.

__________________________________________________

## Competitive Positioning

### FBCA Unique Advantages

**Church-Specific** — Purpose-built for ministry workflows

**Planning Center Integration** — No commercial platform offers this

**Cost** — 80-90% cheaper than commercial alternatives

**Customization** — Can add ministry-specific features

**Data Ownership** — Full control, no vendor lock-in

### Gaps to Close

**Mobile Experience** — Table stakes in 2026

**Emergency Features** — Safety/regulatory requirement

**Modern UI** — Expectation for ease of use

**Integration Ecosystem** — APIs enable automation

### Recommended Strategy

Maintain the church-specific focus while incrementally adding commercial-grade features. Don't try to be Brivo or Verkada — be the best door control system FOR CHURCHES, with features that matter for ministry:

Prayer room scheduling integration

Ministry team access management

Volunteer check-in correlation

Youth group/event-specific access

Integration with church management systems

__________________________________________________

## Key Takeaways

**Mobile credentials are non-negotiable** — Build the app first

**Emergency lockdown is a safety requirement** — Prioritize immediately

**Touchless entry is expected post-pandemic** — Wave-to-unlock differentiates

**Cloud architecture reduces long-term costs** — 20-30% savings vs on-prem

**Integration ecosystem matters** — APIs enable automation

**Maintain church focus** — Don't compete with Brivo on features, compete on fit

**UI/UX investment pays off** — Modern design improves adoption

**Video integration is powerful** — Leverage existing Spot.ai investment

__________________________________________________

## Next Steps

**Review & prioritize** this plan with leadership

**Secure budget** for Phase 1 development ($30-45K)

**Hire mobile developer** or engage agency for iOS/Android app

**Design mockups** for new dashboard UI

**Plan rollout** — pilot with select doors before full deployment

__________________________________________________

*Research compiled from: Brivo, Avigilon/Openpath, Verkada, Kisi, RemoteLock, HID Global, ButterflyMX, and industry market reports from Allied Market Research, Mordor Intelligence, and MarketsandMarkets.*

Document prepared by: Cornerstone (FBCA AI Operations Manager)

Research date: March 2-3, 2026

Status: Ready for leadership review